UCT41042 - PRACTICAL FOR INTRODUCTION TO SMART SYSTEMS

Continous Assignment

Reg.No:SEU/IS/20/ICT/084                                                                                                 Date:2026/06/04

Q01.

Circuit design in Wokwi:

Code:

#define BLYNK_TEMPLATE_ID "TMPL6q3adVhDC"

#define BLYNK_TEMPLATE_NAME "Smart Parking Assistant"

#define BLYNK_AUTH_TOKEN "qewiSQH7C5NzYRaUZ18ZYXtl_wXvXZKv"

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

// WiFi details

char ssid[] = "Wokwi-GUEST";

char pass[] = "";

// Sensor pins

#define TRIG_PIN 5

#define ECHO_PIN 18

// RGB LED pins

#define RED_LED 25

#define GREEN_LED 26

#define BLUE_LED 27

// Buzzer pin

#define BUZZER 14

void setup()

{

// Start serial monitor

Serial.begin(115200);

// System start message

Serial.println("SMART PARKING ASSISTANT");

Serial.println("System Testing Started");

// Set sensor pins

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

// Set LED pins

pinMode(RED_LED, OUTPUT);

pinMode(GREEN_LED, OUTPUT);

pinMode(BLUE_LED, OUTPUT);

// Set buzzer pin

pinMode(BUZZER, OUTPUT);

// Connect to WiFi and Blynk

Serial.println("Connecting...");

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// Check Blynk connection

if (Blynk.connected())

{

Serial.println("Blynk Connected");

}

else

{

Serial.println("Blynk Not Connected");

}

}

void loop()

{

// Run Blynk

Blynk.run();

long duration;

float distance;

// Send ultrasonic signal

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read echo signal

duration = pulseIn(ECHO_PIN, HIGH);

// Calculate distance

distance = duration * 0.034 / 2;

// Show distance

Serial.print("Distance : ");

Serial.print(distance);

Serial.println(" cm");

// Send distance to Blynk

Blynk.virtualWrite(V0, distance);

// Danger zone

if (distance < 10){

// Red LED ON

digitalWrite(RED_LED, HIGH);

digitalWrite(GREEN_LED, LOW);

digitalWrite(BLUE_LED, LOW);

// Fast buzzer

digitalWrite(BUZZER, HIGH);

delay(100);

digitalWrite(BUZZER, LOW);

delay(100);

// Update Blynk

Blynk.virtualWrite(V1, 255);

Blynk.virtualWrite(V2, 0);

Blynk.virtualWrite(V3, 0);

Blynk.virtualWrite(V4, "DANGER");

Serial.println("Status : DANGER");

}

// Caution zone

else if (distance >= 10 && distance <= 30){

// Yellow LED ON

digitalWrite(RED_LED, HIGH);

digitalWrite(GREEN_LED, HIGH);

digitalWrite(BLUE_LED, LOW);

// Slow buzzer

digitalWrite(BUZZER, HIGH);

delay(500);

digitalWrite(BUZZER, LOW);

delay(500);

// Update Blynk

Blynk.virtualWrite(V1, 0);

Blynk.virtualWrite(V2, 255);

Blynk.virtualWrite(V3, 0);

Blynk.virtualWrite(V4, "CAUTION");

Serial.println("Status : CAUTION");

}

// Safe zone

else{

// Green LED ON

digitalWrite(RED_LED, LOW);

digitalWrite(GREEN_LED, HIGH);

digitalWrite(BLUE_LED, LOW);

// Buzzer OFF

digitalWrite(BUZZER, LOW);

// Update Blynk

Blynk.virtualWrite(V1, 0);

Blynk.virtualWrite(V2, 0);

Blynk.virtualWrite(V3, 255);

Blynk.virtualWrite(V4, "SAFE");

Serial.println("Status : SAFE");

delay(500);

}

}

Serial Monitor output:

Blynk Web Dashboard:

Q02.

Circuit design in Wokwi:

Code:

#define BLYNK_TEMPLATE_ID "TMPL6Sc1JV5Xb"

#define BLYNK_TEMPLATE_NAME "Intrusion Detection System"

#define BLYNK_AUTH_TOKEN "rXIb-8A1eqDXVzU7wYKu-1nfId4IGYfI"

#include <Arduino.h>

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

#include <SPI.h>

#include <MFRC522.h>

char ssid[] = "Wokwi-GUEST";

char pass[] = "";

#define PIR_PIN 13

#define BUZZER 14

#define DOOR_RELAY 27

#define SS_PIN 5

#define RST_PIN 22

MFRC522 rfid(SS_PIN, RST_PIN);

bool intrusion = false;

unsigned long intrusionTime = 0;

void setup()

{

Serial.begin(115200);

pinMode(PIR_PIN, INPUT);

pinMode(BUZZER, OUTPUT);

pinMode(DOOR_RELAY, OUTPUT);

SPI.begin();

rfid.PCD_Init();

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

Serial.println("System Ready");

}

void loop()

{

Blynk.run();

int motion = digitalRead(PIR_PIN);

if (motion == HIGH && intrusion == false)

{

intrusion = true;

intrusionTime = millis();

digitalWrite(BUZZER, HIGH);

Serial.println("Intrusion Alert!");

Blynk.virtualWrite(V0, 255);

}

if (intrusion == true)

{

if (rfid.PICC_IsNewCardPresent() &&

rfid.PICC_ReadCardSerial())

{

String uid = "";

for (byte i = 0; i < rfid.uid.size; i++)

{

uid += String(rfid.uid.uidByte[i], HEX);

}

Serial.print("Card UID: ");

Serial.println(uid);

// Replace with your RFID UID

if (uid == "1234")

{

digitalWrite(DOOR_RELAY, HIGH);

Blynk.virtualWrite(V1, "Access Granted");

Serial.println("Access Granted");

digitalWrite(BUZZER, LOW);

intrusion = false;

delay(5000);

digitalWrite(DOOR_RELAY, LOW);

}

}

if (millis() - intrusionTime > 10000)

{

Blynk.virtualWrite(V1, "Access Denied");

Serial.println("Access Denied");

digitalWrite(BUZZER, LOW);

Blynk.virtualWrite(V0, 0);

intrusion = false;

}

}

}

Serial Monitor output:

Blynk Web Dashboard: