UCT41042 - PRACTICAL FOR INTRODUCTION TO SMART SYSTEMS

Assignment 02

Reg.No:SEU/IS/20/ICT/084                                                                                     Name : P.R.P.S.Kalhara

Q01

Question 1 – Circuit

Question 1 – Code

#include <Arduino.h>

#include <WiFi.h>

#include <PubSubClient.h>

#define TRIG_PIN 5

#define ECHO_PIN 18

#define PUMP_PIN 2

#define WIFI_SSID "Wokwi-GUEST"

#define WIFI_PASSWORD ""

#define MQTT_BROKER "192.168.116.178"

#define MQTT_PORT 1883

#define MQTT_CLIENT "esp32-water-tank"

WiFiClient espClient;

PubSubClient mqtt(espClient);

const char *TOPIC_LEVEL = "home/tank/level";

const char *TOPIC_PUMP = "home/tank/pump";

const char *TOPIC_CMD = "home/tank/pump/set";

bool manualMode = false;

void callback(char *topic, byte *payload, unsigned int length){

String msg = "";

for (int i = 0; i < length; i++)

msg += (char)payload[i];

if (String(topic) == TOPIC_CMD){

if (msg == "ON"){

digitalWrite(PUMP_PIN, HIGH);

mqtt.publish(TOPIC_PUMP, "ON", true);

}

if (msg == "OFF"){

digitalWrite(PUMP_PIN, LOW);

mqtt.publish(TOPIC_PUMP, "OFF", true);

}

}

}

void connectWiFi(){

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED){

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected");

}

void connectMQTT(){

while (!mqtt.connected()){

Serial.print("Connecting MQTT...");

if (mqtt.connect(MQTT_CLIENT)){

Serial.println("Connected");

mqtt.subscribe(TOPIC_CMD);

}else{

delay(2000);

}

}

}

float getDistance(){

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH);

float distance = duration * 0.034 / 2;

return distance;

}

void setup(){

Serial.begin(115200);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(PUMP_PIN, OUTPUT);

connectWiFi();

mqtt.setServer(MQTT_BROKER, MQTT_PORT);

mqtt.setCallback(callback);

}

void loop(){

if (!mqtt.connected())

connectMQTT();

mqtt.loop();

static unsigned long lastPub = 0;

if (millis() - lastPub > 2000){

float distance = getDistance();

float level = ((200 - distance) / 200.0) * 100;

if (level < 0)

level = 0;

if (level > 100)

level = 100;

Serial.print("Distance: ");

Serial.print(distance);

Serial.print(" cm  ");

Serial.print("Level: ");

Serial.print(level);

Serial.println("%");

mqtt.publish(TOPIC_LEVEL,String((int)level).c_str(),true);

if (level < 30){

digitalWrite(PUMP_PIN, HIGH);

mqtt.publish(TOPIC_PUMP, "Pump ON - Low Water Level",true);

}else if (level > 80){

digitalWrite(PUMP_PIN, LOW);

mqtt.publish(TOPIC_PUMP,"Tank Full",true);

}else{

digitalWrite(PUMP_PIN, LOW);

mqtt.publish(TOPIC_PUMP,"Normal Level",true);

}

lastPub = millis();

}

}

Question 1– Serial Monitor

Question 1 – Node-RED Dashboard

Q02

Question 2 – Circuit

Question 2 – Code

#include <Arduino.h>

#include <WiFi.h>

#include <PubSubClient.h>

#define POT_PIN 34

#define LOAD1_PIN 15

#define LOAD2_PIN 18

#define WIFI_SSID "Wokwi-GUEST"

#define WIFI_PASSWORD ""

#define MQTT_BROKER "192.168.116.178"

#define MQTT_PORT 1883

#define MQTT_CLIENT "esp32-energy-monitor"

WiFiClient espClient;

PubSubClient mqtt(espClient);

const char *TOPIC_POWER = "home/energy/power";

const char *TOPIC_LOAD1 = "home/energy/load1";

const char *TOPIC_LOAD2 = "home/energy/load2";

const char *TOPIC_ALERT = "home/energy/alert";

const char *TOPIC_LOAD1_CMD = "home/energy/load1/set";

const char *TOPIC_LOAD2_CMD = "home/energy/load2/set";

bool autoShutdown = false;

void callback(char* topic, byte* payload, unsigned int length) {

String msg="";

for(int i=0;i<length;i++){

msg += (char)payload[i];

}

if(!autoShutdown){

if(String(topic)==TOPIC_LOAD1_CMD){

if(msg=="ON"){

digitalWrite(LOAD1_PIN,HIGH);

mqtt.publish(TOPIC_LOAD1,"ON",true);

}else{

digitalWrite(LOAD1_PIN,LOW);

mqtt.publish(TOPIC_LOAD1,"OFF",true);

}

}

if(String(topic)==TOPIC_LOAD2_CMD){

if(msg=="ON"){

digitalWrite(LOAD2_PIN,HIGH);

mqtt.publish(TOPIC_LOAD2,"ON",true);

}else{

digitalWrite(LOAD2_PIN,LOW);

mqtt.publish(TOPIC_LOAD2,"OFF",true);

}

}

}

}

void connectWiFi(){

WiFi.begin(WIFI_SSID,WIFI_PASSWORD);

while(WiFi.status()!=WL_CONNECTED){

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected");

}

void connectMQTT(){

while(!mqtt.connected()){

Serial.print("Connecting MQTT...");

if(mqtt.connect(MQTT_CLIENT)){

Serial.println("Connected");

mqtt.subscribe(TOPIC_LOAD1_CMD);

mqtt.subscribe(TOPIC_LOAD2_CMD);

}else{

delay(2000);

}

}

}

void setup(){

Serial.begin(115200);

pinMode(LOAD1_PIN,OUTPUT);

pinMode(LOAD2_PIN,OUTPUT);

connectWiFi();

mqtt.setServer(MQTT_BROKER,MQTT_PORT);

mqtt.setCallback(callback);

}

void loop(){

if(!mqtt.connected()){

connectMQTT();

}

mqtt.loop();

static unsigned long lastPub=0;

if(millis()-lastPub > 1000){

int rawValue = analogRead(POT_PIN);

int powerPercent = map(rawValue,0,4095,0,100);

Serial.print("Raw Value: ");

Serial.print(rawValue);

Serial.print("  Power: ");

Serial.print(powerPercent);

Serial.println("%");

mqtt.publish(TOPIC_POWER,String(powerPercent).c_str(),true);

if(powerPercent > 65){

autoShutdown = true;

digitalWrite(LOAD1_PIN,LOW);

mqtt.publish(TOPIC_LOAD1,"OFF",true);

mqtt.publish(TOPIC_ALERT,"High Power - Load 1 OFF",true);

}else{

autoShutdown = false;

mqtt.publish(TOPIC_ALERT,"Normal Power Usage",true);

}

mqtt.publish(TOPIC_LOAD1,digitalRead(LOAD1_PIN) ? "ON":"OFF",true);

mqtt.publish(TOPIC_LOAD2,digitalRead(LOAD2_PIN) ? "ON":"OFF",true);

lastPub = millis();

}

}

Question 2– Serial Monitor

Question 2 – Node-RED Dashboard