Lab Sheet 10

Title: IoT Sensor Data Logging System using ESP32 and Google Sheets.

Aims:

- To design and implement an IoT-based data logging system using an ESP32 microcontroller and a temperature sensor, where real-time sensor data is transmitted over Wi-Fi and stored in a Google Sheets spreadsheet. This system demonstrates cloud-based data storage, real-time monitoring, and basic IoT integration using HTTP requests and Google Apps Script.

Task 01: IoT Sensor Data Logging System Setup

- Project Setup in Wokwi / VS Code

  - Name the project SensorData_ESP32_GoogleSheet.

- Circuit Design

  - Sensor Connection

    - Connect the DS18B20 (One-Wire Digital Temperature Sensor) to the ESP32.

- Library Integration

  - WiFi (Built-in ESP32 library)

  - HTTPClient (Built-in ESP32 library)

  - OneWire (External library)

  - DallasTemperature (External library)

- Google Sheets Configuration

  - Create a new Google Sheet

  - Name the sheet ESP32_Sensor_Data

  - Create the following column headers:

    - Date

    - Time

    - Sensor Value

  - Open Google Apps Script from the Extensions menu

  - Write a script to receive HTTP GET/POST requests from ESP32

var sheet = SpreadsheetApp.getActiveSpreadsheet().getActiveSheet();

function doGet(e)

{

var date = new Date();

var sensor = e.parameter.temp;

sheet.appendRow([

Utilities.formatDate(date,"GMT+5:30","yyyy-MM-dd"),

Utilities.formatDate(date,"GMT+5:30","HH:mm:ss"),

sensor

]);

return ContentService.createTextOutput("Success");

}

  - Deploy the script as a Web App

  - Set access permission to Anyone

  - Copy the generated Web App URL

https://script.google.com/macros/s/AKfycbwHEwHLWSVRaAjUB69jsKUGvT_aW2AcFX_MicxC9c9yrRh5XBHwBtY4Y-JMR38Uok4r/exec

- Programming Logic Development in Wokwi / VS Code

  - Initialize Serial Monitor

  - Initialize Wi-Fi connection

  - Initialize the DS18B20 sensor

  - Read temperature values at regular intervals

  - Send sensor data to Google Sheets using an HTTP requestData should be sent automatically at a fixed time interval (e.g., every 2 seconds)

    - Each sensor reading should be stored as a new row in Google Sheets

Code:

#include <Arduino.h>

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <HTTPClient.h>

#include <OneWire.h>

#include <DallasTemperature.h>

const char* ssid = "Wokwi-GUEST";

const char* password = "";

String scriptURL = "https://script.google.com/macros/s/AKfycbwHEwHLWSVRaAjUB69jsKUGvT_aW2AcFX_MicxC9c9yrRh5XBHwBtY4Y-JMR38Uok4r/exec";

// DS18B20 Data Pin

#define ONE_WIRE_BUS 4

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

void setup()

{

Serial.begin(115200);

sensors.begin();

WiFi.begin(ssid, password);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi Connected");

}

void loop()

{

sensors.requestTemperatures();

float temp = sensors.getTempCByIndex(0);

Serial.print("Temperature : ");

Serial.print(temp);

Serial.println(" °C");

if (WiFi.status() == WL_CONNECTED)

{

WiFiClientSecure client;

client.setInsecure();

HTTPClient http;

String url = scriptURL + "?temp=" + String(temp);

http.setFollowRedirects(HTTPC_FORCE_FOLLOW_REDIRECTS);

http.begin(client, url);

int httpCode = http.GET();

Serial.print("HTTP Response : ");

Serial.println(httpCode);

http.end();

}

delay(2000);

}

- Simulation & Testing in Wokwi / VS Code

  - Simulate the circuit in Wokwi

  - Verify temperature readings in the Serial Monitor

  - Confirm real-time data entries in Google Sheets