UCT41042 - PRACTICAL FOR INTRODUCTION TO SMART SYSTEMS

CA-03

Reg.No:SEU/IS/20/ICT/084						Name:P.R.P.S.Kalhara

Diagram

Code:

#include <Arduino.h>

#include <WiFi.h>

#include <ESPSupabase.h>

#include <LiquidCrystal_I2C.h>

#include <time.h>

#define WIFI_SSID "Wokwi-GUEST"

#define WIFI_PASSWORD ""

#define SUPABASE_URL "https://mnyjdwffbiqwiznupecp.supabase.co"

#define SUPABASE_API_KEY "eyJhbGciOiJIUzI1NiIsInR5cCI6IkpXVCJ9.eyJpc3MiOiJzdXBhYmFzZSIsInJlZiI6Im1ueWpkd2ZmYmlxd2l6bnVwZWNwIiwicm9sZSI6ImFub24iLCJpYXQiOjE3ODIyODE3MDQsImV4cCI6MjA5Nzg1NzcwNH0.ioUkA_sK996QbTqYOcRmir0Pvr_kxnHpHDoh9BwJd20"

LiquidCrystal_I2C lcd(0x27, 16, 2);

#define LDR_PIN 34

#define RED_PIN 19

#define GREEN_PIN 18

#define BLUE_PIN 17

#define STREET_LED_PIN 5

Supabase supabase;

unsigned long lastSend = 0;

const unsigned long interval = 5000;

bool blinkState = false;

void connectWiFi();

void setRGB(bool r, bool g, bool b);

void syncTime();

int sendToSupabase(String jsonStr);

void setup()

{

Serial.begin(115200);

Serial.println("\n=== SMART STREET LIGHT SYSTEM STARTED ===");

pinMode(RED_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

pinMode(BLUE_PIN, OUTPUT);

pinMode(STREET_LED_PIN, OUTPUT);

lcd.init();

lcd.backlight();

lcd.print("SMART STREET");

lcd.setCursor(0, 1);

lcd.print("LIGHT SYSTEM");

delay(2000);

lcd.clear();

connectWiFi();

// Sync time before any HTTPS/TLS calls

syncTime();

// Initialize Supabase

supabase.begin(SUPABASE_URL, SUPABASE_API_KEY);

}

void loop()

{

int ldrValue = analogRead(LDR_PIN);

int intensity = map(ldrValue, 0, 4095, 0, 100);

intensity = constrain(intensity, 0, 100);

String status = "OFF";

String ledState = "UNKNOWN";

if (intensity > 70) {

setRGB(false, true, false);

digitalWrite(STREET_LED_PIN, LOW);

ledState = "Green";

}

else if (intensity >= 30) {

setRGB(false, false, true);

digitalWrite(STREET_LED_PIN, LOW);

ledState = "BLUE";

}

else if (intensity >= 15) {

setRGB(true, false, false);

digitalWrite(STREET_LED_PIN, HIGH);

ledState = "RED";

status = "ON";

}

else {

blinkState = !blinkState;

setRGB(blinkState, false, false);

digitalWrite(STREET_LED_PIN, HIGH);

status = "ON";

ledState = "RED_BLINK";

}

lcd.setCursor(0, 0);

lcd.print("Light: ");

lcd.print(intensity);

lcd.print("%   ");

lcd.setCursor(0, 1);

lcd.print("Street: ");

lcd.print(status);

lcd.print("   ");

if (millis() - lastSend >= interval) {

lastSend = millis();

// Build JSON payload

JsonDocument doc;

doc["intensity"] = intensity;

doc["status"] = status;

doc["ledState"] = ledState;

String jsonStr;

serializeJson(doc, jsonStr);

int responseCode = sendToSupabase(jsonStr);

Serial.print("Supabase Response Code: ");

Serial.println(responseCode);

}

}

int sendToSupabase(String jsonStr) {

int responseCode = -1;

const int maxAttempts = 3;

for (int attempt = 1; attempt <= maxAttempts; attempt++) {

responseCode = supabase.insert("street_light_logs", jsonStr, false);

if (responseCode > 0) {

// (HTTP 2xx/etc) — stop retrying

break;

}

Serial.print("Insert attempt ");

Serial.print(attempt);

Serial.print(" failed (code ");

Serial.print(responseCode);

Serial.println("), retrying...");

delay(1000);

}

return responseCode;

}

void connectWiFi() {

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi CONNECTED!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

}

void syncTime() {

configTime(0, 0, "pool.ntp.org", "time.nist.gov");

Serial.print("Syncing time");

time_t now = time(nullptr);

while (now < 1700000000) {

delay(500);

Serial.print(".");

now = time(nullptr);

}

Serial.println("\nTime synced: " + String(now));

}

void setRGB(bool r, bool g, bool b) {

digitalWrite(RED_PIN, r);

digitalWrite(GREEN_PIN, g);

digitalWrite(BLUE_PIN, b);

}

Output:

Database:

Mobile App: