Lab Sheet 01

Supportive Document for VS code

Title: Create and Run ESP32 Project using Wokwi Simulator and platformIO extension in Visual Studio Code.

Part A: Setup Wokwi in Visual Studio Code

- Install Required Extensions in VS Code

  - Open Visual Studio Code.

  - Go to Extensions.

  - Install the following:

    - Wokwi Simulator

    - PlatformIO

- Activate Wokwi License

  - Press F1 to open Command Palette.

  - Type and select: Wokwi: Request a new License

  - Click Open when prompted.

  - It will open your browser. Log in to your Wokwi account.

  - Click GET YOUR LICENSE.

  - Return to VS Code and confirm the license activation.

Part B: Create a New ESP32 Project in VS Code 1. Open the PlatformIO sidebar (ant icon on the left).

- Click on New Project.

- Fill in the details:

  - Name: ESP32-FirstProject

  - Board: Espressif ESP32 Dev Module

  - Framework: Arduino

  - Choose your desired location → Click Finish.

- Wait until PlatformIO finishes initializing the project.

Part C: Add Wokwi Simulation Support

After the project is created, you need to add two important files for Wokwi:

- In the root of your project, create a new file named wokwi.toml and paste the following: [wokwi] version = 1 firmware = ".pio/build/esp32dev/firmware.bin" elf = ".pio/build/esp32dev/firmware.elf"

- Create a diagram.json file in the root to define hardware.

- Right click diagram.json file and select open with, then select Text Editor.

- Copy and paste here, the code from digram.json file in Wokwi site.

Part D: Add the code for ESP32

- Open the file:

  - src/main.cpp

- Remove default codes except #include <Arduino.h>

- Copy and paste here, the code from sketch.ino file in Wokwi site.

- Save the file.

Part E: Build and Run the Simulation

- Press f1 → Click on PlatformIO:Build → (or press the ✓ icon at the bottom).

- After successful build, press F1 → type and select: Wokwi: Start Simulator

How to Add Libraries for ESP32 in Visual Studio Code using PlatformIO

Method: Using PlatformIO Library Manager

- In the PlatformIO sidebar, click on PIO Home → Open.

- In the PlatformIO Home page, click on the Libraries tab (on the left menu).

- In the search bar, type the name of the library you want to add.

  - Example: Type WiFiManager

- Find the correct library in the results (for WiFiManager, choose tzapu/WiFiManager).

- Click the Add to Project button.

- Select your current project from the list and click Add.

- PlatformIO will automatically add the library to your platformio.ini file.

- After adding the library, you can use it in your code like this:

    - #include <WiFiManager.h>

    - WiFiManager wm;

Lab Sheet No: 01

Title: Getting Started with ESP32 – Digital Input/Output and Wi-Fi Connectivity Basics

Aims: Introducing the fundamentals of ESP32 programming by developing and simulating digital input/output and Wi-Fi connectivity applications, including establishing Wi-Fi connections using both hardcoded credentials and the WiFiManager library, while using the Serial Monitor for debugging and real-time observation.

Tasks:

- Create and simulate ESP32 projects in the Wokwi simulator.

- Use the Serial Monitor for debugging and real-time observation.

- Connect the ESP32 to Wi-Fi using hardcoded SSID and password.

- Implement dynamic Wi-Fi configuration using the WiFiManager library.

- Combine Wi-Fi status with LED behavior for visual feedback and print connection details (IP address) only after successful connection.

# ESP32

Exercise 1: Connecting ESP32 to Wi-Fi

Part A: Hardcoded Wi-Fi Connection (Simple Method)

- Create a new ESP32 project in Wokwi.

- Write the code for hardcoded Wi-Fi connection using Wokwi-GUEST (open network, no password).

- Run the simulation and observe the Serial Monitor (connection progress dots followed by successful connection and IP address).

- Change the SSID to an incorrect name (e.g., "MyHomeWiFi") and observe the connection failure.

#include <Arduino.h>

#include <WiFi.h>

const char* ssid = "Wokwi-GUEST";  // FIXED

const char* password = "";

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\nConnecting to WiFi...");

Serial.println(ssid);

WiFi.mode(WIFI_STA); // optional but good practice

WiFi.begin(ssid, password); // FIXED

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 50) {

Serial.print(".");

delay(500);  // IMPORTANT FIX

attempts++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\nConnected Successfully!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

} else {

Serial.println("\nFailed to connect to WiFi");

}

}

void loop() {

}

Part B: Using WiFiManager (Recommended for Real Projects)

- Add the WiFiManager library in Wokwi (via Library Manager).

- Write and run the WiFiManager code.

- Observe the Serial Monitor for portal creation and connection status.

- (For real hardware) Connect your phone/laptop to the “ESP32-Config-Portal” Access Point, open a browser at 192.168.4.1, and enter your Wi-Fi SSID and password.

#include <WiFi.h>

#include <WiFiManager.h>

WiFiManager wm;

void setup() {

Serial.begin(115200);

delay(10000);

Serial.println("\n\n ESP32 WiFi Manager Starting...");

WiFi.mode(WIFI_STA);

bool res = wm.autoConnect("ESP32-Config-Portal");

if(res){

Serial.println("Succesfully connected to Wi-Fi");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

}else{

Serial.println("\nFailed to connect.");

}

}

void loop() {

delay(1000);

}

Part C: Combine Wi-Fi status with LED behavior for visual feedback.

- Blink the onboard LED slowly when not connected to Wi-Fi.

- Blink the LED fast or keep it ON when successfully connected.

- Print the IP address only after a successful connection.

#include <WiFi.h>

#include <WiFiManager.h>

WiFiManager wm;

#define LED_PIN 2   // ESP32 onboard LED (usually GPIO 2)

bool wasConnected = false;

bool ledState = false;

unsigned long previousMillis = 0;

const unsigned long DISCONNECTED_BLINK_MS = 1000;

const unsigned long CONNECTED_BLINK_MS = 200;

void blinkLed(unsigned long intervalMs) {

if (millis() - previousMillis >= intervalMs) {

previousMillis = millis();

ledState = !ledState;

digitalWrite(LED_PIN, ledState);

}

}

void setup() {

Serial.begin(115200);

delay(2000);

pinMode(LED_PIN, OUTPUT);

Serial.println("\n\nESP32 WiFi Manager Starting...");

WiFi.mode(WIFI_STA);

// Keep loop running while the portal is active so LED can blink.

wm.setConfigPortalBlocking(false);

// Start WiFi Manager (AP portal will open if no WiFi saved)

wm.autoConnect("ESP32-Config-Portal");

}

void loop() {

wm.process();

const bool isConnected = (WiFi.status() == WL_CONNECTED);

if (!isConnected) {

// Not connected -> slow blink

blinkLed(DISCONNECTED_BLINK_MS);

wasConnected = false;

} else {

// Connected -> fast blink

blinkLed(CONNECTED_BLINK_MS);

// Print IP only when connection is first established

if (!wasConnected) {

Serial.println("\nWi-Fi Connected Successfully!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

wasConnected = true;

}

}

}