Lab Sheet 02

Title: Over-The-Air (OTA) Firmware Updates with ESP32

Aims: To introduce the concept, importance, and implementation of Over-The-Air (OTA) updates on the ESP32 platform. Students will learn how to enable wireless firmware updates, perform updates remotely and handle OTA events in IoT projects.

Tasks:

- Understand the OTA process and its benefits (remote maintenance, no physical access needed).

- Implement OTA using the ArduinoOTA library.

- Monitor and handle OTA events (start, progress, end, errors).

- Perform version updates and verify success.

- Explore visual feedback.

Key Concepts

- OTA: Firmware update delivered wirelessly (usually over Wi-Fi).

- Partitions: ESP32 uses separate OTA partitions to safely swap between old and new firmware.

- Workflow: Device connects to Wi-Fi → Listens for update → Downloads new binary → Validates → Reboots into new firmware.

- Advantages: Easy field updates, scalability for multiple devices.

- Risks: Interrupted updates, incompatible firmware, security vulnerabilities.

Exercise 1: Basic OTA Setup with ArduinoOTA

- Create a new ESP32 project in Wokwi or open in Arduino IDE.

- Connect a LED with a 220Ω current-limiting resistor to GPIO 2 (Pin 2) of the ESP32.

- Write a sketch that connects to Wi-Fi and enables ArduinoOTA.

- Include proper error handling and event callbacks.

- Upload the initial version (call it Version 1) and observe the Serial Monitor.

Expected Output in Serial Monitor:

#include <Arduino.h>

#include <WiFi.h>

#include <ArduinoOTA.h>

// ─── Configuration ────────────────────────────────────────────────

const char* WIFI_SSID     = "Kalara-WiFi";  // Replace with your Wi-Fi SSID

const char* WIFI_PASSWORD = "Pasa2001#";   // Replace with your Wi-Fi password

const char* FIRMWARE_VERSION = "1.0.0";

const char* HOSTNAME         = "ESP32-OTA-v1";

const int LED_PIN       = 2;      // Built-in LED / GPIO 2 via 220Ω resistor

const int BLINK_DELAY   = 1000;   // 1 second blink interval (V1 pattern)

void printBanner();

void connectToWiFi();

void setupOTA();

void blinkLED();

// ─── Setup ────────────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

delay(500);

pinMode(LED_PIN, OUTPUT);

printBanner();

connectToWiFi();

setupOTA();

Serial.println("✅ System ready. Waiting for OTA updates...");

Serial.println("─────────────────────────────────────────────");

}

// ─── Loop ─────────────────────────────────────────────────────────

void loop() {

ArduinoOTA.handle();   // Must be called in every loop iteration

blinkLED();            // V1: slow blink (1 second on/off)

}

// ─── Wi-Fi Connection ─────────────────────────────────────────────

void connectToWiFi() {

Serial.printf("\n📡 Connecting to Wi-Fi: %s\n", WIFI_SSID);

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

attempts++;

if (attempts > 40) {

Serial.println("\n❌ Wi-Fi connection failed! Restarting...");

ESP.restart();

}

}

Serial.println("\n✅ Wi-Fi Connected!");

Serial.printf("   SSID      : %s\n", WiFi.SSID().c_str());

Serial.printf("   IP Address: %s\n", WiFi.localIP().toString().c_str());

Serial.printf("   Signal    : %d dBm\n", WiFi.RSSI());

}

// ─── OTA Configuration ────────────────────────────────────────────

void setupOTA() {

ArduinoOTA.setHostname(HOSTNAME);

// Optional: set a password for OTA security

// ArduinoOTA.setPassword("admin");

// ── OTA Event: Start ──────────────────────────────────────────

ArduinoOTA.onStart([]() {

String type = (ArduinoOTA.getCommand() == U_FLASH) ? "Sketch" : "Filesystem";

Serial.println("\n🚀 OTA Update Started!");

Serial.printf("   Update type: %s\n", type.c_str());

digitalWrite(LED_PIN, LOW); // LED off during update

});

// ── OTA Event: End ────────────────────────────────────────────

ArduinoOTA.onEnd([]() {

Serial.println("\n✅ OTA Update Complete! Rebooting...");

Serial.println("─────────────────────────────────────────────");

});

// ── OTA Event: Progress ───────────────────────────────────────

ArduinoOTA.onProgress([](unsigned int progress, unsigned int total) {

unsigned int percent = (progress / (total / 100));

Serial.printf("\r⬆️  Uploading: [");

int bars = percent / 5;                        // 20 bars total

for (int i = 0; i < 20; i++) {

Serial.print(i < bars ? "█" : "░");

}

Serial.printf("] %3u%%", percent);

});

// ── OTA Event: Error ──────────────────────────────────────────

ArduinoOTA.onError([](ota_error_t error) {

Serial.printf("\n❌ OTA Error [%u]: ", error);

switch (error) {

case OTA_AUTH_ERROR:    Serial.println("Authentication Failed"); break;

case OTA_BEGIN_ERROR:   Serial.println("Begin Failed");          break;

case OTA_CONNECT_ERROR: Serial.println("Connect Failed");        break;

case OTA_RECEIVE_ERROR: Serial.println("Receive Failed");        break;

case OTA_END_ERROR:     Serial.println("End Failed");            break;

default:                Serial.println("Unknown Error");         break;

}

});

ArduinoOTA.begin();

Serial.println("\n📥 OTA Service Started");

Serial.printf("   Hostname  : %s\n", HOSTNAME);

Serial.printf("   OTA Port  : 3232 (default)\n");

}

// ─── LED Blink — V1 Pattern (Slow: 1s on / 1s off) ───────────────

void blinkLED() {

static unsigned long lastToggle = 0;

static bool ledState = false;

if (millis() - lastToggle >= BLINK_DELAY) {

lastToggle = millis();

ledState = !ledState;

digitalWrite(LED_PIN, ledState);

}

}

// ─── Banner ───────────────────────────────────────────────────────

void printBanner() {

Serial.println("─────────────────────────────────────────────");

Serial.println("   ESP32 OTA Firmware Update Demo");

Serial.printf("   Firmware Version : %s\n", FIRMWARE_VERSION);

Serial.printf("   Compiled         : %s %s\n", __DATE__, __TIME__);

Serial.println("─────────────────────────────────────────────");

}

Exercise 2: Performing Your First OTA Update

- Modify the initial sketch to create Version 2:

- Change the version message.

- Modify LED blink pattern (e.g., faster blink or different duty cycle).

- Add a new feature (e.g., print “OTA Update Successful – Now Running v2”).

- In Arduino IDE:

- Select the network port (ESP32’s IP address).

- Upload the new sketch over-the-air.

- Observe the update progress, completion message, and automatic reboot.

- Verify the new behavior (LED pattern + Serial output).

Tools → Port → Select Network Port → [ESP32-OTA-v1 at 192.168.x.x] → Sketch → Upload   (or Ctrl+U)

#include <Arduino.h>

#include <WiFi.h>

#include <ArduinoOTA.h>

// ─── Configuration ────────────────────────────────────────────────

const char* WIFI_SSID     = "Kalara-WiFi";  // Replace with your Wi-Fi SSID

const char* WIFI_PASSWORD = "Pasa2001#";   // Replace with your Wi-Fi password

const char* FIRMWARE_VERSION = "2.0.0";        // ← Version bumped

const char* HOSTNAME         = "ESP32-OTA-v2"; // ← Hostname updated

const int LED_PIN = 2;

// V2 Blink pattern: fast triple-flash then pause

const int FLASH_ON    = 100;   // 100ms on

const int FLASH_OFF   = 100;   // 100ms between flashes

const int PAUSE_TIME  = 800;   // 800ms pause after triple-flash

// ─── Function Prototypes ──────────────────────────────────────────

void printBanner();

void connectToWiFi();

void setupOTA();

void blinkLEDv2();

// ─── Setup ────────────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

delay(500);

pinMode(LED_PIN, OUTPUT);

printBanner();

connectToWiFi();

setupOTA();

// ── V2 Success Message ────────────────────────────────────────

Serial.println("─────────────────────────────────────────────");

Serial.println("🎉 OTA Update Successful – Now Running v2!");

Serial.println("   New features in this version:");

Serial.println("   • Triple-flash LED blink pattern");

Serial.println("   • Improved OTA progress display");

Serial.println("   • Version tracking in Serial output");

Serial.println("─────────────────────────────────────────────");

}

// ─── Loop ─────────────────────────────────────────────────────────

void loop() {

ArduinoOTA.handle();   // Must be called in every loop iteration

blinkLEDv2();          // V2: fast triple-flash pattern

}

// ─── Wi-Fi Connection ─────────────────────────────────────────────

void connectToWiFi() {

Serial.printf("\n📡 Connecting to Wi-Fi: %s\n", WIFI_SSID);

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

attempts++;

if (attempts > 40) {

Serial.println("\n❌ Wi-Fi connection failed! Restarting...");

ESP.restart();

}

}

Serial.println("\n✅ Wi-Fi Connected!");

Serial.printf("   SSID      : %s\n", WiFi.SSID().c_str());

Serial.printf("   IP Address: %s\n", WiFi.localIP().toString().c_str());

Serial.printf("   Signal    : %d dBm\n", WiFi.RSSI());

}

// ─── OTA Configuration ────────────────────────────────────────────

void setupOTA() {

ArduinoOTA.setHostname(HOSTNAME);

// ── OTA Event: Start ──────────────────────────────────────────

ArduinoOTA.onStart([]() {

String type = (ArduinoOTA.getCommand() == U_FLASH) ? "Sketch" : "Filesystem";

Serial.println("\n🚀 OTA Update Started!");

Serial.printf("   Update type: %s\n", type.c_str());

Serial.printf("   Current version: %s → New version incoming...\n", FIRMWARE_VERSION);

digitalWrite(LED_PIN, LOW);

});

// ── OTA Event: End ────────────────────────────────────────────

ArduinoOTA.onEnd([]() {

Serial.println("\n✅ OTA Transfer Complete!");

Serial.println("   Validating firmware...");

Serial.println("   Rebooting into new firmware now...");

Serial.println("─────────────────────────────────────────────");

});

// ── OTA Event: Progress ───────────────────────────────────────

ArduinoOTA.onProgress([](unsigned int progress, unsigned int total) {

unsigned int percent   = (progress / (total / 100));

unsigned int received  = progress / 1024;

unsigned int totalKB   = total    / 1024;

Serial.printf("\r⬆️  Uploading: [");

int bars = percent / 5;

for (int i = 0; i < 20; i++) {

Serial.print(i < bars ? "█" : "░");

}

Serial.printf("] %3u%% (%u / %u KB)", percent, received, totalKB);

});

// ── OTA Event: Error ──────────────────────────────────────────

ArduinoOTA.onError([](ota_error_t error) {

Serial.printf("\n❌ OTA Error [%u]: ", error);

switch (error) {

case OTA_AUTH_ERROR:    Serial.println("Authentication Failed – Check password"); break;

case OTA_BEGIN_ERROR:   Serial.println("Begin Failed – Not enough space?");        break;

case OTA_CONNECT_ERROR: Serial.println("Connect Failed – Network issue?");         break;

case OTA_RECEIVE_ERROR: Serial.println("Receive Failed – Upload interrupted");     break;

case OTA_END_ERROR:     Serial.println("End Failed – Validation error");           break;

default:                Serial.println("Unknown Error");                            break;

}

Serial.println("   ⚠️  Device remains on current firmware.");

});

ArduinoOTA.begin();

Serial.println("\n📥 OTA Service Started");

Serial.printf("   Hostname  : %s\n", HOSTNAME);

Serial.printf("   OTA Port  : 3232 (default)\n");

}

// ─── LED Blink — V2 Pattern (Triple-flash then pause) ─────────────

void blinkLEDv2() {

static unsigned long lastAction = 0;

static int step = 0;

unsigned long now = millis();

// Step sequence: ON, OFF, ON, OFF, ON, PAUSE (repeat)

//   0: flash 1 ON   → FLASH_ON

//   1: flash 1 OFF  → FLASH_OFF

//   2: flash 2 ON   → FLASH_ON

//   3: flash 2 OFF  → FLASH_OFF

//   4: flash 3 ON   → FLASH_ON

//   5: pause (LED off) → PAUSE_TIME

const int delays[] = {

FLASH_ON, FLASH_OFF,

FLASH_ON, FLASH_OFF,

FLASH_ON, PAUSE_TIME

};

const bool states[] = {

HIGH, LOW,

HIGH, LOW,

HIGH, LOW

};

if (now - lastAction >= (unsigned long)delays[step]) {

lastAction = now;

digitalWrite(LED_PIN, states[step]);

step = (step + 1) % 6;

}

}

// ─── Banner ───────────────────────────────────────────────────────

void printBanner() {

Serial.println("─────────────────────────────────────────────");

Serial.println("   ESP32 OTA Firmware Update Demo");

Serial.printf("   Firmware Version : %s\n", FIRMWARE_VERSION);

Serial.printf("   Compiled         : %s %s\n", __DATE__, __TIME__);

Serial.println("─────────────────────────────────────────────");

}

Enter Wi-Fi Password