Lab Sheet 04

Title: Interfacing Input & Output Devices with ESP32 in Wokwi

Aims: Learn how to correctly connect common Input Devices and Output Devices to the ESP32 Devkit in the Wokwi online simulator. This lab sheet focuses only on hardware connections (wiring) and includes a brief explanation of each device’s purpose.

Tasks:

- Open wokwi and start a new project with ESP32 DevKit.

- Use the + button to search and add each component.

- Use different colored wires (Red = Power, Black = GND, others for signals).

- ESP32 GPIO pins are 3.3V logic. Always connect GND properly..

Input Devices

# 1. Pushbutton (Tactile Switch)

A momentary switch used to detect manual user input (press). Commonly used for triggering actions, resetting systems, or simple user interfaces.

#include <Arduino.h>

// Define the pins

const int BUTTON_PIN = 5;  // GIOP5 connected to button

const int LED_PIN    = 2;  // Onboard LED

// variable for storing the pushbutton status

int buttonState = 0;

void setup() {

Serial.begin(115200);

// Initialize the pushbutton pin as an input with internal pull-up

// This means the pin is HIGH by default and LOW when pressed

pinMode(BUTTON_PIN, INPUT_PULLUP);

// Initialize the LED pin as an output

pinMode(LED_PIN, OUTPUT);

}

void loop() {

// Read the state of the pushbutton value

buttonState = digitalRead(BUTTON_PIN);

// Check if the pushbutton is pressed.

// If it is, the buttonState is LOW (because of INPUT_PULLUP)

if (buttonState == LOW) {

digitalWrite(LED_PIN, HIGH); // Turn LED on

Serial.println("Button Pressed!");

} else {

digitalWrite(LED_PIN, LOW);  // Turn LED off

}

}

# 2. Slide Switch / Toggle Switch

Maintains ON/OFF state for mode selection, power control, or configuration settings.

#include <Arduino.h>

// Define the pins

const int SWITCH_PIN = 5;

const int LED_PIN    = 2; // Internal LED

void setup() {

Serial.begin(115200);

// Set the switch pin as input with internal pull-up

pinMode(SWITCH_PIN, INPUT_PULLUP);

// Set the onboard LED as output

pinMode(LED_PIN, OUTPUT);

Serial.println("System Initialized. Flip the switch!");

}

void loop() {

// Read the current position of the switch

int switchState = digitalRead(SWITCH_PIN);

if (switchState == LOW) {

// Switch is closed (connected to GND)

digitalWrite(LED_PIN, HIGH);

Serial.println("Status: ON");

} else {

// Switch is open (HIGH due to pull-up)

digitalWrite(LED_PIN, LOW);

Serial.println("Status: OFF");

}

// Small delay to prevent flooding the Serial Monitor

delay(200);

}

# 3.	Potentiometer (Rotary)

Variable resistor used to adjust analog values (e.g., volume, brightness, speed control).

#include <Arduino.h>

// ADC1_CH6 is GPIO 34

const int POT_PIN = 34;

void setup() {

Serial.begin(115200);

// Configure ADC resolution (optional, default is 12-bit)

analogReadResolution(12);

Serial.println("Potentiometer Read Initialized...");

}

void loop() {

// Read the analog value (0 - 4095)

int rawValue = analogRead(POT_PIN);

// Convert the raw value to a voltage (0 - 3.3V)

float voltage = (rawValue * 3.3) / 4095.0;

// Print the results

Serial.print("Raw Value: ");

Serial.print(rawValue);

Serial.print(" | Voltage: ");

Serial.print(voltage);

Serial.println("V");

// Small delay for readability

delay(250);

}

# 4. Rotary Encoder (KY-040)

Detects rotation direction and steps. Used for menu navigation, volume control, and precise parameter adjustment.

#include <Arduino.h>

// Define pins

#define CLK_PIN 18

#define DT_PIN  19

#define SW_PIN  5

int counter = 0;

int currentStateCLK;

int lastStateCLK;

unsigned long lastButtonPress = 0;

void setup() {

Serial.begin(115200);

// Setup pins

pinMode(CLK_PIN, INPUT);

pinMode(DT_PIN, INPUT);

pinMode(SW_PIN, INPUT_PULLUP);

// Read the initial state of CLK

lastStateCLK = digitalRead(CLK_PIN);

}

void loop() {

// 1. Read Rotation

currentStateCLK = digitalRead(CLK_PIN);

// If CLK has changed, a pulse occurred

if (currentStateCLK != lastStateCLK && currentStateCLK == 1) {

// If the DT state is different from the CLK state,

// the encoder is rotating CCW

if (digitalRead(DT_PIN) != currentStateCLK) {

counter--;

} else {

// Encoder is rotating CW

counter++;

}

Serial.print("Direction: ");

Serial.print(digitalRead(DT_PIN) != currentStateCLK ? "CCW" : "CW");

Serial.print(" | Count: ");

Serial.println(counter);

}

lastStateCLK = currentStateCLK;

// 2. Read Button Press

int btnState = digitalRead(SW_PIN);

if (btnState == LOW) {

// Basic debouncing

if (millis() - lastButtonPress > 50) {

Serial.println("Button Pressed!");

}

lastButtonPress = millis();

}

// No delay here! We need high-speed polling for the encoder.

}

# 5. Analog Joystick

Provides analog X & Y axis movement and a digital button. Used in gaming, robotics control, and pantilt systems.

#include <Arduino.h>

// Define pins

const int PIN_X = 34;

const int PIN_Y = 35;

const int PIN_SW = 5;

void setup() {

Serial.begin(115200);

// Joystick button needs a pull-up

pinMode(PIN_SW, INPUT_PULLUP);

// Set ADC resolution to 12-bit (0-4095)

analogReadResolution(12);

}

void loop() {

// Read analog values

int xValue = analogRead(PIN_X);

int yValue = analogRead(PIN_Y);

// Read button (inverted logic because of INPUT_PULLUP)

int swValue = digitalRead(PIN_SW);

// Print raw values

Serial.print("X: ");

Serial.print(xValue);

Serial.print(" | Y: ");

Serial.print(yValue);

Serial.print(" | Button: ");

Serial.println(swValue == LOW ? "PRESSED" : "RELEASED");

// Logic Example: Simple Direction detection

if (xValue < 1500) Serial.println("Moving LEFT");

else if (xValue > 2500) Serial.println("Moving RIGHT");

if (yValue < 1500) Serial.println("Moving UP");

else if (yValue > 2500) Serial.println("Moving DOWN");

delay(200);

}

# 6.	Keypad

Allows numeric and character input. Used in password systems, calculators, and menu-based projects.

# 7. IR Receiver

Detects and receives infrared signals from an IR Remote. Used for wireless remote control of devices (TVs, robots, home automation, etc.).

#include <Arduino.h>

#include <IRremoteESP8266.h>

#include <IRrecv.h>

#include <IRutils.h>

// 1. Define the GPIO pin connected to the IR sensor

const uint16_t kRecvPin = 4;

// 2. Establish the IR receiver object

IRrecv irrecv(kRecvPin);

decode_results results;

void setup() {

Serial.begin(115200);

// 3. Start the receiver

irrecv.enableIRIn();

Serial.print("IR Receiver ready on GPIO ");

Serial.println(kRecvPin);

}

void loop() {

// 4. Check if a signal was received

if (irrecv.decode(&results)) {

// Print the protocol (NEC, Sony, etc.) and the Hex code

serialPrintUint64(results.value, HEX);

Serial.println("");

// Resume listening for the next signal

irrecv.resume();

}

delay(100);

}

Output Devices

# 1. IR Remote

It acts like a TV remote. It transmits (sends) invisible infrared light signals when you press its buttons.

# 2.	LED

Visual indicator for status, alerts, or debugging.

#include <Arduino.h>

// Define the LED pin

const int ledPin = 5;

void setup() {

// Initialize the digital pin as an output

pinMode(ledPin, OUTPUT);

Serial.begin(115200);

Serial.println("LED Blink Test Started");

}

void loop() {

// Turn the LED on (HIGH is the voltage level)

digitalWrite(ledPin, HIGH);

Serial.println("LED ON");

delay(1000); // Wait for a second

// Turn the LED off by making the voltage LOW

digitalWrite(ledPin, LOW);

Serial.println("LED OFF");

delay(1000); // Wait for a second

}

# 3. RGB LED

Produces multiple colors for status indication or decorative lighting.

#include <Arduino.h>

// Define Pins

const int redPin = 5;

const int greenPin = 18;

const int bluePin = 19;

// PWM Settings

const int freq = 5000;

const int resolution = 8; // 8-bit resolution (0-255)

// PWM Channels

const int redChannel = 0;

const int greenChannel = 1;

const int blueChannel = 2;

const bool commonAnode = false;

// Forward declaration

void setColor(int redValue, int greenValue, int blueValue);

void setup() {

Serial.begin(115200);

// Set pins as outputs

pinMode(redPin, OUTPUT);

pinMode(greenPin, OUTPUT);

pinMode(bluePin, OUTPUT);

// Configure LED PWM functionalites

ledcSetup(redChannel, freq, resolution);

ledcSetup(greenChannel, freq, resolution);

ledcSetup(blueChannel, freq, resolution);

ledcAttachPin(redPin, redChannel);

ledcAttachPin(greenPin, greenChannel);

ledcAttachPin(bluePin, blueChannel);

// Startup check: LED should turn white briefly.

setColor(255, 255, 255);

delay(1000);

setColor(0, 0, 0);

Serial.println("RGB LED initialized!");

}

void loop() {

// Cycle through some colors

Serial.println("RED");

setColor(255, 0, 0);   // Red

delay(1000);

Serial.println("GREEN");

setColor(0, 255, 0);   // Green

delay(1000);

Serial.println("BLUE");

setColor(0, 0, 255);   // Blue

delay(1000);

Serial.println("YELLOW");

setColor(255, 255, 0); // Yellow

delay(1000);

Serial.println("PURPLE");

setColor(255, 0, 255); // Purple

delay(1000);

Serial.println("CYAN");

setColor(0, 255, 255); // Cyan

delay(1000);

Serial.println("WHITE");

setColor(255, 255, 255); // White

delay(1000);

Serial.println("OFF");

setColor(0, 0, 0);     // Off

delay(1000);

}

// Function to set the RGB color

void setColor(int redValue, int greenValue, int blueValue) {

if (commonAnode) {

redValue = 255 - redValue;

greenValue = 255 - greenValue;

blueValue = 255 - blueValue;

}

ledcWrite(redChannel, redValue);

ledcWrite(greenChannel, greenValue);

ledcWrite(blueChannel, blueValue);

}

# 4. Buzzer

Produces simple beeps or tones for alerts, notifications, or feedback.

#include <Arduino.h>

// Define the buzzer pin

const int buzzerPin = 5;

void playMelody() {

int notes[] = {262, 294, 330, 349}; // C4, D4, E4, F4

for (int i = 0; i < 4; i++) {

tone(buzzerPin, notes[i]);

delay(200);

noTone(buzzerPin);

delay(50);

}

}

void setup() {

// No special setup needed for tone()

Serial.begin(115200);

}

void loop() {

Serial.println("Buzzer: ON (1000Hz)");

// Play a 1000Hz tone

tone(buzzerPin, 1000);

delay(500);

Serial.println("Buzzer: OFF");

// Stop the tone

noTone(buzzerPin);

delay(1000);

// Simple melody test (Passive Buzzer only)

playMelody();

}

# 5. Servo Motor

Precise angular control (0–180°). Used in robotics, camera gimbals, and door locks.

#include <Arduino.h>

#include <ESP32Servo.h>

// 1. Create a servo object

Servo myServo;

// 2. Define the GPIO pin

const int servoPin = 18;

void setup() {

Serial.begin(115200);

// 3. Attach the servo to the pin

// Standard servos use a pulse width of 500us to 2400us

myServo.attach(servoPin, 500, 2400);

}

void loop() {

// Move from 0 to 180 degrees

for (int pos = 0; pos <= 180; pos += 1) {

myServo.write(pos);

delay(15); // Wait for the servo to reach the position

}

delay(1000);

// Move from 180 to 0 degrees

for (int pos = 180; pos >= 0; pos -= 1) {

myServo.write(pos);

delay(15);

}

delay(1000);

}

# 6. Bipolar Stepper Motor + A4988 Driver

Provides precise angular movement and position control. Used in robotics, 3D printers, CNC machines, and automated positioning systems.

#include <Arduino.h>

#include <AccelStepper.h>

// 1. Define the pins

const int stepPin = 32;

const int dirPin = 19;

// 2. Define the motor interface type (1 means a driver like A4988)

#define motorInterfaceType 1

// 3. Create the stepper object

AccelStepper stepper(motorInterfaceType, stepPin, dirPin);

void setup() {

Serial.begin(115200);

// 4. Set maximum speed and acceleration

stepper.setMaxSpeed(1000);     // Steps per second

stepper.setAcceleration(500);  // Steps per second squared

Serial.println("Stepper Test Initialized...");

}

void loop() {

// Move to a position (e.g., 200 steps is usually one full rotation)

Serial.println("Moving to 400...");

stepper.moveTo(400);

stepper.runToPosition(); // Blocking call: waits until finished

delay(1000);

// Move back to start

Serial.println("Moving back to 0...");

stepper.moveTo(0);

stepper.runToPosition();

delay(1000);

}

# 7. Relay Module

Allows low-power ESP32 to control high-power devices (lights, motors, appliances).

#include <Arduino.h>

// Define the Relay Pin

const int relayPin = 5;

void setup() {

// Initialize the relay pin as an output

pinMode(relayPin, OUTPUT);

// Set initial state to OFF (for Active Low relays)

digitalWrite(relayPin, HIGH);

Serial.begin(115200);

Serial.println("Relay Test Started");

}

void loop() {

// Turn the relay ON

Serial.println("Relay ON - Device Running");

digitalWrite(relayPin, LOW);

delay(3000); // Keep it on for 3 seconds

// Turn the relay OFF

Serial.println("Relay OFF - Device Stopped");

digitalWrite(relayPin, HIGH);

delay(3000); // Keep it off for 3 seconds

}

# 8. LCD 16x2 (Parallel)

Displays text, numbers, and simple messages (2 rows × 16 characters). Classic and very common in beginner projects.

#include <Arduino.h>

#include <LiquidCrystal.h>

// Initialize the library with the numbers of the interface pins

// RS, E, D4, D5, D6, D7

LiquidCrystal lcd(19, 18, 17, 16, 4, 2);

void setup() {

// Set up the LCD's number of columns and rows:

lcd.begin(16, 2);

// Print a message to the LCD.

lcd.print("ESP32 Test...");

delay(2000);

lcd.clear();

}

void loop() {

// Set the cursor to column 0, line 0

lcd.setCursor(0, 0);

lcd.print("System: Online");

// Set the cursor to column 0, line 1

lcd.setCursor(0, 1);

lcd.print("Uptime: ");

lcd.print(millis() / 1000);

lcd.print("s");

delay(1000);

}

# 9. LCD 16x2 (I2C)

Same as above but uses only 2 wires (easier wiring).

#include <Arduino.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// 1. Set the LCD address to 0x27 for a 16 chars and 2 line display

// If 0x27 doesn't work, try 0x3F

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

// 2. Initialize the LCD

lcd.init();

// Turn on the blacklight

lcd.backlight();

// Print a message

lcd.setCursor(0, 0);

lcd.print("I2C LCD Ready!");

lcd.setCursor(0, 1);

lcd.print("ESP32 Booting...");

delay(2000);

lcd.clear();

}

void loop() {

lcd.setCursor(0, 0);

lcd.print("Scanning...");

// Example: Displaying a value

int sensorValue = analogRead(34);

lcd.setCursor(0, 1);

lcd.print("Value: ");

lcd.print(sensorValue);

lcd.print("    "); // Clear trailing digits

delay(500);

}

# 10. MAX7219 LED Dot Matrix

Displays scrolling text, animations, and patterns (8x8 matrix). Can daisy-chain multiple modules.

#include <Arduino.h>

#include <MD_Parola.h>

#include <MD_MAX72xx.h>

#include <SPI.h>

// 1. Define Hardware Type and Pins

// Use the Parola hardware preset for the Wokwi MAX7219 matrix part

#define HARDWARE_TYPE MD_MAX72XX::PAROLA_HW

#define MAX_DEVICES 4 // Set this to the number of 8x8 matrices you have

#define CS_PIN 5

// 2. Initialize the Parola object

MD_Parola myDisplay = MD_Parola(HARDWARE_TYPE, CS_PIN, MAX_DEVICES);

void setup() {

// 3. Initialize the display

myDisplay.begin();

// Set intensity (0-15)

myDisplay.setIntensity(5);

// Clear the display

myDisplay.displayClear();

// Configure the scrolling message once in setup

myDisplay.displayText("ESP32 IoT TEST", PA_CENTER, 100, 0, PA_SCROLL_LEFT, PA_SCROLL_LEFT);

myDisplay.displayReset();

}

void loop() {

// 4. Keep the configured animation running

if (myDisplay.displayAnimate()) {

myDisplay.displayReset();

}

}

# 11. Seven Segment Display

Displays numbers and limited characters. Widely used in clocks, counters, and scoreboards.

#include <Arduino.h>

// Define pins for segments a, b, c, d, e, f, g

const int segments[7] = {13, 12, 14, 27, 26, 25, 33};

// Binary map for numbers 0-9 (a, b, c, d, e, f, g)

const byte digitMap[10] = {

0b00111111, // 0

0b00000110, // 1

0b01011011, // 2

0b01001111, // 3

0b01100110, // 4

0b01101101, // 5

0b01111101, // 6

0b00000111, // 7

0b01111111, // 8

0b01101111  // 9

};

void displayDigit(int num);

void setup() {

for (int i = 0; i < 7; i++) {

pinMode(segments[i], OUTPUT);

}

}

void loop() {

for (int i = 0; i <= 9; i++) {

displayDigit(i);

delay(1000);

}

}

void displayDigit(int num) {

for (int i = 0; i < 7; i++) {

// Check the bit for each segment

int bit = bitRead(digitMap[num], i);

digitalWrite(segments[i], bit ? LOW : HIGH);

}

}