Lab Sheet 05

Title: Programming ESP32 with Sensors in Wokwi

Aims: To program the ESP32 to interface with various sensors, understand their libraries and communication protocols, and display real-time sensor data on the Serial Monitor.

Tasks:

- HC-SR04 (Ultrasonic Distance Sensor)

- Write a program to measure distance in centimeters and display it on the Serial Monitor every second.

#include <Arduino.h>

const int trigPin = 5;

const int echoPin = 18;

void setup()

{

Serial.begin(9600);

Serial.println("\nHC-SR04 (Ultrasonic Distance Sensor) ");

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

}

void loop()

{

// Send pulse

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Read echo time

long duration = pulseIn(echoPin, HIGH);

// Calculate distance

int distance = duration * 0.034 / 2;

// Print distance

Serial.print("Distance = ");

Serial.print(distance);

Serial.println(" cm");

delay(1000);

}

- DHT22 (Digital Humidity & Temperature Sensor)

- Write a program to read and display both temperature (°C) and humidity (%) every 2 seconds.

#include <Arduino.h>

#include <DHT.h>

#define DHTPIN 4

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

void setup()

{

Serial.begin(115200);

Serial.println("\nDHT22 (Digital Humidity & Temperature Sensor) ");

dht.begin();

}

void loop()

{

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println(" %");

delay(2000);

}

- DS18B20 (One-Wire Digital Temperature Sensor)

- Write a program to read and display the temperature in Celsius.

#include <Arduino.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#define ONE_WIRE_BUS 4

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

void setup() {

Serial.begin(9600);

Serial.println("\nDS18B20 (One-Wire Digital Temperature Sensor) ");

sensors.begin();

}

void loop() {

sensors.requestTemperatures();

float temp = sensors.getTempCByIndex(0);

Serial.print("Temperature: ");

Serial.print(temp);

Serial.println(" °C");

delay(1000);

}

- BMP180 (Barometric Pressure & Temperature Sensor)

- Write a program to display Temperature (°C), Pressure (hPa), and Altitude (meters).

#include <Arduino.h>

#include <Wire.h>

#include <Adafruit_BMP085.h>

Adafruit_BMP085 bmp;

void setup()

{

Serial.begin(115200);

Serial.println("\nBMP180 (Barometric Pressure & Temperature Sensor) ");

bmp.begin();

}

void loop()

{

float temperature = bmp.readTemperature();

float pressure = bmp.readPressure() / 100.0;

float altitude = bmp.readAltitude();

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

Serial.print("Pressure: ");

Serial.print(pressure);

Serial.println(" hPa");

Serial.print("Altitude: ");

Serial.print(altitude);

Serial.println(" m");

Serial.println();

delay(2000);

}

- MPU6050 (3-Axis Accelerometer + Gyroscope + Temperature)

- Write a program to display acceleration (X, Y, Z) and gyroscope (X, Y, Z) values.

#include <Arduino.h>

#include <Wire.h>

#include <MPU6050.h>

MPU6050 mpu;

void setup() {

Serial.begin(115200);

Serial.println("\nMPU6050 (3-Axis Accelerometer + Gyroscope + Temperature) ");

Wire.begin();

mpu.initialize();

}

void loop() {

int16_t ax, ay, az;

int16_t gx, gy, gz;

// Read Accelerometer and Gyroscope values

mpu.getMotion6(&ax, &ay, &az, &gx, &gy, &gz);

Serial.println("Accelerometer");

Serial.print("X: ");

Serial.print(ax);

Serial.print("  Y: ");

Serial.print(ay);

Serial.print("  Z: ");

Serial.println(az);

Serial.println("Gyroscope");

Serial.print("X: ");

Serial.print(gx);

Serial.print("  Y: ");

Serial.print(gy);

Serial.print("  Z: ");

Serial.println(gz);

Serial.println();

delay(1000);

}

- PIR Motion Sensor (Passive Infrared)

- Write a program to detect motion and print “Motion Detected” or “No Motion” continuously.

#include <Arduino.h>

const int pirPin = 5;

void setup()

{

Serial.begin(115200);

Serial.println("\nPIR Motion Sensor (Passive Infrared) ");

pinMode(pirPin, INPUT);

}

void loop()

{

int motion = digitalRead(pirPin);

if (motion == HIGH)

{

Serial.println("Motion Detected");

}

else

{

Serial.println("No Motion");

}

delay(1000);

}

- Photoresistor (LDR) Light Sensor

- Write a program to read and display the analog light intensity value.

#include <Arduino.h>

const int ldrPin = 34;

void setup() {

Serial.begin(115200);

Serial.println("\nPhotoresistor (LDR) Light Sensor ");

}

void loop() {

int lightValue = analogRead(ldrPin);

Serial.print("Light Intensity: ");

Serial.println(lightValue);

delay(1000);

}

- Analog Temperature Sensor (NTC Thermistor)

- Write a program to calculate and display temperature in °C using the thermistor.

#include <Arduino.h>

const int thermistorPin = 34;

void setup() {

Serial.begin(115200);

Serial.println("\nAnalog Temperature Sensor (NTC Thermistor) ");

}

void loop() {

int value = analogRead(thermistorPin);

// Convert analog value to temperature

float temperature = map(value, 0, 1023, -20, 100);

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

delay(1000);

}

- MQ2 Gas Sensor

- Write a program to read and display the gas/smoke level (analog value) every 500ms.

#include <Arduino.h>

const int mq2Pin = 34;

void setup() {

Serial.begin(115200);

Serial.println("MQ2_Gas_Sensor ");

}

void loop() {

int gasValue = analogRead(mq2Pin);

Serial.print("Gas Level: ");

Serial.println(gasValue);

delay(500);

}

- HX711 Load Cell Amplifier

- Write a program to display weight in grams. Include taring (zeroing) at startup.

#include <Arduino.h>

#include "HX711.h"

#define DT 16

#define SCK 17

HX711 scale;

void setup()

{

Serial.begin(115200);

Serial.println("\nHX711 Load Cell Amplifier ");

scale.begin(DT, SCK);

// Tare (zero the scale at startup)

Serial.println("Taring... remove any weight");

delay(2000);

scale.tare();

Serial.println("Ready");

}

void loop()

{

// Set calibration factor (you MUST adjust this for your load cell)

float calibration_factor = -7050;

scale.set_scale(calibration_factor);

float weight = scale.get_units(10); // average of 10 readings

Serial.print("Weight: ");

Serial.print(weight);

Serial.println(" g");

delay(500);

}

- MFRC522 RFID/NFC Reader

- Write a program to read an RFID card and print its UID on the Serial Monitor.

#include <Arduino.h>

#include <SPI.h>

#include <MFRC522.h>

#define SS_PIN 5

#define RST_PIN 22

MFRC522 rfid(SS_PIN, RST_PIN);

void setup() {

Serial.begin(115200);

Serial.println("\nMFRC522 RFID/NFC Reader ");

SPI.begin();

rfid.PCD_Init();

Serial.println("Place RFID card near reader...");

}

void loop() {

if (!rfid.PICC_IsNewCardPresent()) {

return;

}

if (!rfid.PICC_ReadCardSerial()) {

return;

}

Serial.print("UID: ");

for (byte i = 0; i < rfid.uid.size; i++) {

Serial.print(rfid.uid.uidByte[i] < 0x10 ? " 0" : " ");

Serial.print(rfid.uid.uidByte[i], HEX);

}

Serial.println();

rfid.PICC_HaltA();

}