Lab Sheet 07

Title: Temperature-Based Servo Motor Control with Manual Override using ESP32 & Blynk.

Aims:

To design and implement a temperature-based servo motor control system using the ESP32 microcontroller, where the servo motor operates automatically according to temperature variations, while also providing a manual override feature through the Blynk mobile application for real-time user control and monitoring.

# Blynk IoT Platform

Blynk is a cloud-based IoT platform that enables real-time bi-directional communication between microcontrollers and smartphone apps. It uses virtual pins (V0–V255) as data channels. The Blynk library abstracts WebSocket/HTTP calls, allowing the developer to read/write virtual pins with simple function calls.

https://blynk.io

Task 01:

- Project Setup in Wokwi / VS Code

- Circuit Design

  - Connect the DHT sensor to an ESP32 GPIO pin

  - Connect the servo motor to a PWM-capable GPIO pin

  - Connect the LCD (I2C) to the ESP32.

- Library Integration

- Blynk

- DHT sensor library

- Servo

- LiquidCrystal_I2C

[env:esp32dev]

platform = espressif32

board = esp32dev

framework = arduino

lib_deps =

blynkkk/Blynk@^1.3.2

adafruit/DHT sensor library@^1.4.7

madhephaestus/ESP32Servo@^3.2.0

marcoschwartz/LiquidCrystal_I2C@^1.1.4

- Blynk Configuration

a.Create a New Template in Blynk (IoT platform)

Sidebar → Developer Zone → My Templates → Create New Template → Enter Template Name → Select Hardware → Choose Connection Type → Click Create → Add Datastreams → Design Dashboard → Save

b.Assign virtual pins

- Temperature: Units: Degrees

- iHumidity: Units: Percentage, % | Max: 100

- Servo: Units: Degrees, ° | Max: 180

Datastreams → Add New Datastream → Virtual Pin

### 🌡 Temperature

- Virtual Pin → V0

- Name → Temperature

- Data Type → Double / Float

- Units → °C (Degrees)

- Min → -40 (optional)

- Max → 80 (optional)

### 💧 Humidity (iHumidity)

- Virtual Pin → V1

- Name → Humidity

- Data Type → Integer / Double

- Units → % (Percentage)

- Min → 0

- Max → 100

### 🎛 Servo Motor

- Virtual Pin → V2

- Name → Servo

- Data Type → Integer

- Units → ° (Degrees)

- Min → 0

- Max → 180

### 🔁 Final Summary (Quick View)

Datastreams → Add →

- V0 → Temperature (°C)

- V1 → Humidity (%)

- V2 → Servo (0–180°)

c.Create a New Device

Sidebar → Developer Zone → My Devices → + New Device → From Template → Select Template → Enter Device Name → Create Device → Get Auth Token (or Device Credentials) → Save

d.Add widgets:

- Temperature display (Gauge)

- Humidity display (Gauge)

- Servo (Switch)

My Device → Open Dashboard → Edit Mode → + Add Widget → Gauge → Select Temperature (V0) → Set Units °C → Save → Add Widget → Gauge → Select Humidity (V1) → Set Units % → Save → Add Widget → Switch (or Slider) → Select Servo (V2) → Set Range 0–180° → Save

e.Obtain Template ID, Template Name and Auth Token

#define BLYNK_TEMPLATE_ID "TMPL6uxMnzyf7"

#define BLYNK_TEMPLATE_NAME "Task01"

#define BLYNK_AUTH_TOKEN "Jmv5Dwj6JzoPuWPqhYbtjjmdcLusNYy9"

- Programming Logic Development in Wokwi / VS Code

  - Initialize Wi-Fi and Blynk connection

  - Read temperature and humidity from DHT sensor

  - Display the values in LCD

#define BLYNK_TEMPLATE_ID "TMPL6uxMnzyf7"

#define BLYNK_TEMPLATE_NAME "Task01"

#define BLYNK_AUTH_TOKEN "Jmv5Dwj6JzoPuWPqhYbtjjmdcLusNYy9"

#include <Arduino.h>

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

#include <DHT.h>

#include <ESP32Servo.h> // Fixed: Standard Servo.h doesn't work on ESP32, changed to ESP32Servo

#include <LiquidCrystal_I2C.h>

#include <Wire.h>

#define DHTPIN 4

#define DHTTYPE DHT22

#define SERVO_PIN 18

#define VPIN_TEMP V0

#define VPIN_HUM V1

#define VPIN_SERVO V2

char ssid[] = "Wokwi-GUEST";

char pass[] = "";

DHT dht(DHTPIN, DHTTYPE);

LiquidCrystal_I2C lcd(0x27, 16, 2);

Servo myservo;

// Added tracker to replace bad delay(2000) in loop

unsigned long lastTime = 0;

BLYNK_WRITE(VPIN_SERVO)

{

int buttonState = param.asInt();

if (buttonState == 1)

{

myservo.write(90);

}

else

{

myservo.write(0);

}

}

void setup()

{

Serial.begin(115200);

Serial.println("Temperature Auotomation Task01");

lcd.init();

lcd.backlight();

lcd.clear();

// Fixed: WiFi.begin must be called BEFORE checking status loop

WiFi.begin(ssid, pass);

lcd.setCursor(0, 0);

lcd.print("Connecting WIFI");

Serial.print("Connecting WIFI");

int attempt = 0;

while (WiFi.status() != WL_CONNECTED && attempt < 20)

{

delay(500);

lcd.print(".");

Serial.print(".");

attempt++;

}

lcd.clear();

if (WiFi.status() == WL_CONNECTED)

{

lcd.setCursor(0, 0);

lcd.println("WiFi Connected");

Serial.println("\nWiFi Connected");

}

else

{

lcd.setCursor(0, 0);

lcd.println("WiFi Conn Failed");

Serial.println("WiFi Connection Failed");

}

delay(1500);

lcd.clear();

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

dht.begin();

// Fixed: ESP32 PWM timer allocation for servo stability

ESP32PWM::allocateTimer(0);

myservo.setPeriodHertz(50);

myservo.attach(SERVO_PIN, 500, 2400);

myservo.write(0);

lcd.setCursor(0, 0);

lcd.print("System Ready");

Serial.println("System Ready");

delay(1000);

lcd.clear();

}

void loop()

{

Blynk.run();

// Fixed: Using millis() instead of delay(2000) to keep Blynk from disconnecting

if (millis() - lastTime >= 500)

{

lastTime = millis();

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (isnan(temperature) || isnan(humidity))

{

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Sensor Error");

Serial.println("Temperature Or Humidity Read Error !");

return;

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print(temperature, 1);

lcd.print(" C");

lcd.setCursor(0, 1);

lcd.print("Humidity: ");

lcd.print(humidity, 1);

lcd.print("%");

Blynk.virtualWrite(VPIN_TEMP, temperature);

Blynk.virtualWrite(VPIN_HUM, humidity);

}

}

- Simulation & Testing in Wokwi / VS Code

  - Run simulation

  - Adjust temperature values in Wokwi / VS Code

  - Observe servo movement

  - Verify Blynk interaction