Lab Sheet 09

Title: ESP32 IoT System with MQTT and Node-RED Integration

Aims:

To design, simulate, and deploy an ESP32-based IoT system that publishes sensor data via MQTT and visualizes it using Node-RED dashboards. Learners will develop firmware, configure an MQTT broker, build Node-RED flows, and create real-time monitoring dashboards for a Smart Room Environmental Monitor.

Tasks: Smart Room Environmental Monitor

# Part 1: Environment Setup

- Install extensions in VS Code

  - MQTT Explorer – standalone MQTT client for debugging

  - PlatformIO IDE by PlatformIO – compile ESP32 firmware

  - Node.js (LTS) – required for Node-RED runtime

node –version

  - Wokwi by Wokwi – browser-based ESP32 simulator

- Install Node-RED globally via npm:

npm install -g --unsafe-perm node-red

- Install an MQTT Broker (Mosquitto):

  - Download from https://mosquitto.org/download/

  - After install, start the broker:

    - mosquitto -v

  - Create file named mosquitto.conf and save on working folder.

    - Type below codes:

listener 1883

allow_anonymous true

  - Start the broker.

    - In cmd : mosquitto -c mosquitto.conf -v

# Part 2: Project Setup in Wokwi / VS Code

- Name the project ESP32NodeRed-Lab.

- Circuit Design

- Connect the DHT22 sensor to an ESP32 GPIO pin.

- Connect the PIR sensor to an ESP32 GPIO pin.

- Connect the LED to the ESP32.

- Library Integration

- PubSubClient

- DHT sensor library

[env:esp32dev]

platform = espressif32

board = esp32dev

framework = arduino

lib_deps =

adafruit/DHT sensor library@^1.4.7

knolleary/PubSubClient@^2.8

- Configure Node-RED

  - Launch Node-RED

  - Install Required Nodes

i. @flowfuse/node-red-dashboard

- Build the MQTT Subscriber Flow

| Node | Type | Configuration |
| --- | --- | --- |
|  | Temperature Flow |  |
| mqtt in | Input | Topic: home/room/temperature, Server: ip_address:1883 |
| gauge | Dashboard | Label: Temperature, Min: 0, Max: 80, Unit: °C |
| chart | Dashboard | Label: Temperature History |
|  | Humidity Flow |  |
| mqtt in | Input | Topic: home/room/humidity Server: ip_address:1883 |
| gauge | Dashboard | Label: Humidity, Min: 0, Max: 100, Unit: % |
|  | Motion Detection Flow |  |
| mqtt in | Input | Topic: home/room/motion |
| text | Dashboard | Label: Motion Status |
|  | LED Control Flow |  |
| switch | Dashboard | Label: LED Control, On Payload: ON, Off Payload: OFF |
| mqtt out | Output | Topic: home/room/led/set, Server: ip_address:1883 |

- Click Deploy (top-right red button) after wiring all nodes.

- Open the dashboard at http://localhost:1880/dashboard to view live data.

- Programming Logic Development in Wokwi / VS Code

  - Initialize Wi-Fi and MQTT connection to enable network communication between ESP32 and the broker.

  - Read temperature and humidity values from the DHT22 sensor using the appropriate sensor

library.

  - Read and process PIR sensor data for motion detection.

  - Publish all sensor readings to MQTT topics in real time for Node-RED dashboard visualization.

#include <Arduino.h>

#include <WiFi.h>

#include <PubSubClient.h>

#include <DHT.h>

// Wi-Fi සහ MQTT Configurations

const char* ssid = "Wokwi-GUEST";

const char* password = "";

const char* mqtt_server = "192.168.116.232";

// Pin Definitions

#define DHTPIN 23

#define DHTTYPE DHT22

#define PIR_PIN 19

#define LED_PIN 22

// MQTT Topics

#define TOPIC_TEMP "home/room/temperature"

#define TOPIC_HUMID "home/room/humidity"

#define TOPIC_MOTION "home/room/motion"

#define TOPIC_LED_SET "home/room/led/set"

DHT dht(DHTPIN, DHTTYPE);

WiFiClient espClient;

PubSubClient client(espClient);

unsigned long lastMsg = 0;

void setup_wifi() {

delay(10);

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi connected");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

}

// MQTT හරහා පණිවිඩ ලැබෙන විට ක්‍රියාත්මක වන ශ්‍රිතය (Callback)

void callback(char* topic, byte* payload, unsigned int length) {

Serial.print("Message arrived [");

Serial.print(topic);

Serial.print("] ");

String message = "";

for (int i = 0; i < length; i++) {

message += (char)payload[i];

}

Serial.println(message);

// LED එක පාලනය කිරීම

if (String(topic) == TOPIC_LED_SET) {

if (message == "ON") {

digitalWrite(LED_PIN, HIGH);

Serial.println("LED turned ON");

} else if (message == "OFF") {

digitalWrite(LED_PIN, LOW);

Serial.println("LED turned OFF");

}

}

}

void reconnect() {

while (!client.connected()) {

Serial.print("Attempting MQTT connection...");

String clientId = "ESP32Client-";

clientId += String(random(0xffff), HEX);

if (client.connect(clientId.c_str())) {

Serial.println("connected");

// LED Topic එක Subscribe කිරීම

client.subscribe(TOPIC_LED_SET);

} else {

Serial.print("failed, rc=");

Serial.print(client.state());

Serial.println(" try again in 5 seconds");

delay(5000);

}

}

}

void setup() {

Serial.begin(115200);

pinMode(PIR_PIN, INPUT);

pinMode(LED_PIN, OUTPUT);

dht.begin();

setup_wifi();

client.setServer(mqtt_server, 1883);

client.setCallback(callback);

}

void loop() {

if (!client.connected()) {

reconnect();

}

client.loop();

unsigned long now = millis();

if (now - lastMsg > 2000) {

lastMsg = now;

// DHT22 දත්ත කියවීම

float h = dht.readHumidity();

float t = dht.readTemperature();

int motionDetected = digitalRead(PIR_PIN);

if (isnan(h) || isnan(t)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Temp: "); Serial.print(t);

Serial.print(" | Humid: "); Serial.print(h);

Serial.print(" | Motion: "); Serial.println(motionDetected ? "MOTION!" : "No Motion");

client.publish(TOPIC_TEMP, String(t).c_str());

client.publish(TOPIC_HUMID, String(h).c_str());

client.publish(TOPIC_MOTION, motionDetected ? "Motion Detected" : "Clear");

}

}

- Simulation & Testing in Wokwi / VS Code

  - Run simulation

  - Adjust values in Wokwi / VS Code

  - Verify Node-RED interaction